## What the original C++ code implements

The C++ source is a software simulation of a NeuroMem-style neural network. It does not use TensorFlow. Instead, it implements an incremental pattern-learning classifier based on committed neurons.

Each neuron stores:

- A context

- A vector of up to 256 byte values

- An Active Influence Field, or AIF

- A minimum influence field

- A category

- A neuron identifier

- The calculated distance from the current input

The network supports three distance calculations:

- L1, or Manhattan distance

- LSUP, or maximum-component distance

- L2, or Euclidean distance

It also supports two classifier behaviours:

- RBF mode: only neurons whose distance falls within their AIF respond.

- KNN mode: all matching-context neurons are considered and ordered by distance.

Learning is incremental. When a vector is learned, the network either:

- Uses an existing neuron of the same category if that neuron already covers the vector.

- Reduces the AIF of conflicting neurons.

- Commits a new neuron if no existing same-category neuron covers the vector.

## MicroPython library design

The MicroPython library provides both Python-style and C++-style method names.

For example, these are equivalent:

ai.learn(vector, category)

ai.Learn(vector, category)

The main supported methods include:

forget()

reset_chain()

size_network()

set_context()

get_context()

set_rbf_classifier()

set_knn_classifier()

set_classifier_type()

enable_l2_norm()

disable_l2_norm()

learn()

classify()

read_neuron()

read_neurons()

write_neurons()

count_committed_neurons()

The register-style interface is also provided:

ai.Read(register)

ai.Write(register, value)

For normal UIFlow 2.0 applications, the high-level learn() and classify() methods are recommended.

## Memory optimisation for M5StickS3

The C++ version allocates memory for all 1,024 neurons at startup:

Neuron = (NEURON*) calloc(networkSize, sizeof(NEURON));

Doing the same with Python objects would consume excessive memory in MicroPython. The created library therefore allocates memory only when a neuron is committed.

For M5StickS3 projects, a smaller network should normally be created:

from nmaicore import NMAICore

ai = NMAICore(network_size=64, vector_size=6)

This example allows up to 64 learned neurons, with each neuron containing six input features.

## Basic use in UIFlow 2.0

Upload nmaicore.py to the M5StickS3 filesystem. It should be placed in the same directory as the main program, usually /flash.

Then import it:

from nmaicore import (NMAICore, NORM_L1, CLASSIFIER_RBF)

Create the classifier:

ai = NMAICore(network_size=64, vector_size=6)

Configure its learning context:

ai.set_context(context=1, norm=NORM_L1, minif=2, maxif=255)

ai.set_classifier_type(CLASSIFIER_RBF)

## Learning examples

All vector components must be byte values between 0 and 255.

VACANT_NORMAL = 1

OCCUPIED_NORMAL = 2

STEAMY_HUMID = 3

POOR_AIR = 4

Example training vectors:

ai.learn(

bytearray([

64,    # Temperature

102,   # Humidity

13,    # Gas response

128,   # Light

0,     # Motion

0      # Occupancy

]),

VACANT_NORMAL

)

ai.learn(

bytearray([

115,

128,

26,

115,

89,

255

]),

OCCUPIED_NORMAL

)

ai.learn(

bytearray([

166,

217,

51,

89,

140,

255

]),

STEAMY_HUMID

)

ai.learn(

bytearray([

140,

166,

230,

102,

77,

179

]),

POOR_AIR

)

## Classifying a vector

sample = bytearray([160, 210, 45, 90, 130, 255])

result = ai.classify(sample)

The returned result is a dictionary:

print("Category:", result["category"])

print("Distance:", result["distance"])

print("Neuron ID:", result["nid"])

print("Identified:", result["identified"])

print("Uncertain:", result["uncertain"])

Example structure:

{

"nid": 3,

"distance": 28,

"category": 3,

"raw_category": 3,

"degenerated": False,

"status": 8,

"identified": True,

"uncertain": False

}

## Multiple classification results

KNN mode can return several nearest neurons:

ai.set_knn_classifier()

results = ai.classify(sample, k=3)

for result in results:

print(result["category"], result["distance"], result["nid"])

## Converting smart-bathroom readings into bytes

The library expects byte vectors, so the bathroom sensor readings must be normalised into the range from 0 to 255.

def clamp_byte(value):

value = int(value)

if value < 0:

return 0

if value > 255:

return 255

return value

def build_bathroom_vector():

temperature_value = clamp_byte(((temperature - 20.0) / 20.0) * 255)

humidity_value = clamp_byte((humidity / 100.0) * 255)

if (gas_baseline > 0 and gas_resistance > 0):

gas_response = 1.0 - (gas_resistance / gas_baseline)

else:

gas_response = 0.0

gas_value = clamp_byte(gas_response * 255)

light_value = clamp_byte((light_lux / 1000.0) * 255)

motion_value = clamp_byte((average_motion_activity / 10.0) * 255)

occupancy_value = (255 if occupied else 0)

return bytearray([

temperature_value,

humidity_value,

gas_value,

light_value,

motion_value,

occupancy_value

])

The live classification can then be performed using:

bathroom_vector = build_bathroom_vector()

result = ai.classify(bathroom_vector)

predicted_category = result["category"]

distance = result["distance"]

## Saving and restoring learned neurons

The learned network can be exported:

neuron_data = ai.read_neurons()

The returned format follows the C++ record layout:

Context,

Vector components,

AIF,

MINIF,

Category

The data may be written to a file:

import json

with open(

"/flash/bathroom_model.json",

"w"

) as file:

json.dump(neuron_data, file)

To restore the model:

with open(

"/flash/bathroom_model.json",

"r"

) as file:

neuron_data = json.load(file)

ai.write_neurons(neuron_data)

For frequent saving, a binary format would be more storage-efficient than JSON.

## Differences and corrections from the C++ implementation

The MicroPython version retains the main learning and classification behaviour but makes several deliberate implementation changes.

The original C++ LSUP condition contains:

(Neuron[RTLIndex].Context & NORM) == 1

Since NORM is 0x0080, the masked result would normally be 0x0080, not 1. This means that branch may not execute as intended. The MicroPython library checks the norm bit correctly.

The original L2 implementation applies the square root only when:

CompIndex == VECTORSIZE - 1

This means vectors shorter than 256 components may not receive the final square-root operation. The MicroPython implementation calculates Euclidean distance correctly for the actual input-vector length.

The C++ result-reading mechanism uses register operations that mutate the firing list as DIST, CAT, and NID are read. The Python classify() method instead returns structured result dictionaries, which is easier and safer to use in UIFlow 2.0.

Low-level serial-register neuron writing is not fully reproduced through repeated Write() operations. Complete neuron models should be imported using:

ai.write_neurons(data)

This avoids fragile chain-state operations and is more suitable for MicroPython.

## Recommended configuration for the smart bathroom

For six sensor features:

ai = NMAICore(network_size=64, vector_size=6)

ai.set_context(context=1, norm=NORM_L1, minif=2, maxif=255)

maxif=255 is a reasonable starting value for a six-component byte vector, but it should be tuned using real training and validation data. For L1 distance, the theoretical maximum distance for six components is 1,530, so a larger maximum influence field may be required if the classes vary significantly.